# PID Control Example 2: Design Steps

| Step | Example 4 |
| --- | --- |
| 1: Collect design specifications Transient: tr, ts, %OS Steady-state: final values (yss, ess) with respect to reference and/or disturbance input |  |
| 1.5: Select a controller type Proportional (P): Proportional-derivative (PD): Proportional-integral (PI): PID: |  |
| 2: Find all necessary closed-loop transfer functions (depend on design specifications), including plugging in your G(s) and C(s) |  |
| 3: Translate design specifications into requirements on n, , ess, and yss, then match equations to find control gains KP, KI, and KD. (a) find the desired closed-loop transfer function and match to the actual closed-loop transfer function (b) find the actual steady state error or output and match to the desired value |  |