Function: 0.5(s+0.5)(s+1)(s+0.1)

| Gain Margin | Phase Margin |
| --- | --- |
| At what frequency does phase equal -180°? 0.7 rad/sec | At what frequency does magnitude equal 0dB? 0.6 rad/sec |
| At this frequency, what is the magnitude (in dB)? -5dB | What is the phase at this frequency? -160° |
| Compute the gain margin as number of dB below 0dB. 5dB = 20log(GM) GM = 1.8 | Compute phase margin as number of degrees above -180° 20° |

Function: 0.5e-s20s+0.5(s+1)(s+0.1)

| Gain Margin | Phase Margin |
| --- | --- |
| At what frequency does phase equal -180°? 7 rad/sec | At what frequency does magnitude equal 0dB? 5 rad/sec |
| At this frequency, what is the magnitude (in dB)? -10dB | What is the phase at this frequency? -160° |
| Compute the gain margin as number of dB below 0dB. 10dB = 20log(GM) GM = 3.2 | Compute phase margin as number of degrees above -180° 20° |

Function: 20,000s+20,000s4+303s3+20,902s2+60,600s+40,000

| Gain Margin | Phase Margin |
| --- | --- |
| At what frequency does phase equal -180°? 130 rad/sec | At what frequency does magnitude equal 0dB? 3 rad/sec |
| At this frequency, what is the magnitude (in dB)? -50dB | What is the phase at this frequency? 110° |
| Compute the gain margin as number of dB below 0dB. 50dB = 20log(GM) GM = 316 | Compute phase margin as number of degrees above -180° 70° |

Function: 1s(s+1)

| Gain Margin | Phase Margin |
| --- | --- |
| At what frequency does phase equal -180°? ω→∞ | At what frequency does magnitude equal 0dB? 0.8 rad/sec |
| At this frequency, what is the magnitude (in dB)? ω→-∞ | What is the phase at this frequency? -130° |
| Compute the gain margin as number of dB below 0dB. GM→∞ | Compute phase margin as number of degrees above -180° 50° |