Analytic solution using plot function in Matlab

Found with Matlab stepplot function for 3 different values of k

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

% Chris Coulston      Spring 2023    Colorado School of Mines

% EENG 307  Intro to Feedback Control Systems

% A mathematical of a Honda Element in the frequency domain.

% All measurements are in SI units.

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

% The equation for x follows from 3 pages of math and couple of hours

t = 0:pi/100:67*pi;

x = 10 - 10.*exp(-t/20).*cos(0.0866.*t)- 5.77.*exp(-t/20).*sin(0.0866.*t);

plot(t,x)

xlabel('Time (seconds)');

ylabel('Position (m)');

title('Position vs. Time (Honda Element) parking with P controller');

waitforbuttonpress;

close all;

s = tf('s');

m = 1000;       % units of kilograms

b = 100;        % drag

kp = 10;

X = 1/(m*s^2+ b*s);         % Transfer function of X/F for my Honda Element

X_k1   = feedback(1*X,1);   % Transfer function for closed loop

X_k10  = feedback(10*X,1);   % Transfer function for closed loop

X_k100 = feedback(100*X,1);   % Transfer function for closed loop

stepplot(10*X_k1,10*X_k10, 10*X_k100, 200);     % Plot the response for a step input

grid on;

xlabel('Time');

ylabel('Position (m)');

title('Position vs. Time (Honda Element) parking with P controller');

legend('k=1', 'k=10', 'k=100');

waitforbuttonpress;

close all;